Ogallala aquifer groundwater-buffering project

# Purpose and holdings

Satellite, reanalysis, model-assimilated and in-situ data over the aquifer (boundary HPA_polygon/hp_bound2010) were assembled to quantify how groundwater buffers vegetation and evapotranspiration (ET) in drought through a Temporal Buffering Index, B(t) = F(t) − F	(t), where F is the ET exp anomaly and F	 the anomaly expected from rain, root-zone soil moisture, exp

VPD, radiation, temperature, land cover and season; a static vulnerability index VI = DSI / (α·SBI + β·GBI) sits beside it. Products live in merged_datasets/ .Files are standalone CF NetCDF/GeoTIFF at native resolution; sources are read-only and every derived product is a new file.

# Data processing (22 Colab notebooks + 18 local scripts)

All variables described in MERGED_DATASETS_CATALOG_2026-10-22.md

| Weather, drought | gridMET daily 1979–2026-10-05 and drought indices (SPI/EDDI/…) from the lab's own files on the native 287×233 (1/24°) grid, packed int16; replaced the Earth Engine copies after the audit. Zarr copies of gridMET, drought, SMAP, SSEBop, MCD12Q1. |
| --- | --- |

ET, energy	MODIS/SSEBop dekadal ET; ECOSTRESS ET (ECO_L3T_JET, 70 m tiles); ECHO-ET (UNH); OpenET v2.1 monthly 30 m (ensemble + 6 models, version switch to v2.0).

Carbon, SIF	OCO-2 and OCO-3 L2 Lite SIF soundings; GOSIF 8-day/monthly with its extraction mesh.

Water, soil	SMAP L4 soil moisture (9 km) and L3 VOD; moisture	GRACE/GRACE-FO storage; HRES water-table depth; HiHydroSoil groups; gSSURGO stack (120 m) and plant-available water.

Vegetation,	MOD13A3 vegetation indices, MCD12Q1 land cover, GLO land	DEM, GDE map, GFSAD1000 cropland, global irrigation

areas, USDA CDL (2008→, on the 30 m Landsat tiles), LANID irrigation (30 m annual 1997–2017 + derived layers), GEDI L4B biomass (1 km).

Wells, towers USGS well networks → water-level-change grids (2017–19, predevelopment–19), hydraulic conductivity, wells time series; AmeriFlux/NEON tower records.

How	Google Colab + Earth Engine exports for most; local Python (SSM env) for USGS, GOSIF, gSSURGO, clipped GDE and drought-fit products. Every notebook checks downloads, reads files back, resumes after a stop, and records its settings in the output.

# Landsat vegetation indices, OpenET, CDL

- 30 m monthly (Landsat_Indices_30m_Monthly): Landsat 5/7/8/9

Collection 2 Level-2 Tier 1, 18 indices (NDVI, EVI, EVI2, SAVI, MSAVI,

OSAVI, GNDVI, NIRv, kNDVI, NDMI, NMDI, MSI, GVMI, NDWI, MNDWI, AWEInsh, AWEIsh, WI2015). Per pixel the monthly median of all passes of a UTM zone, no resampling, 5120-px tiles, yearly files <year>/<tile>/, int16 with scale factors, zlib 2. Choosable start/stop month; cloud, saturation, valid-range and aerosol screens; Landsat 7 cut at 2016; TM/ETM+→OLI harmonisation switch (on); extra variables n_obs, obs_day, n_obs_oli, NDVI/NDMI range. Verified by independent recomputation and on real Earth Engine windows; a full-record run is not documented here.

- OpenET and CDL are placed on the same 30 m tiles as these files (pixel for pixel); LANID keeps its own 30 m Albers grid, which is the CDL's.

# ET residual modelling (residual_et package, document of 7 Oct)

Two models estimate the expected ET anomaly whose residual is B: a cross-fitted XGBoost (25 inputs, 10 land-cover fractions, conformalised 5/50/95 % quantiles) and a differentiable HBV water balance whose 15 parameters come from a 2-layer LSTM (815,887 weights). Neither sees water-table depth or irrigation. Evaluation uses 5-fold rotation over half-year blocks (70/20/10 train/test/validation, dry/normal/wet balanced) with an autocorrelation-based embargo.

- Corrected package: 24 Python files (sources, grid, climate, drought definitions, splits, uncertainty, both trainers, buffering statistics, tower comparison), 41 tests pass (as of 7 Oct); a 144-cell smoke tile runs prepare → XGBoost → LSTM (train, resume, infer) → buffering.

| XGBoost | New LSTM |
| --- | --- |
| 800 trees, 6 depth, | 256 x 2 blocks, 822K params, single-component HBV |
| RMSE: 10.44 mm/ 10 day | RMSE: 15.10 mm/day |
| R² = 0.654 | R² = 0.30 |

- Tower check (run on real data): SSEBop vs 21 flux towers, 2,797 site-dekads, RMSE 12.0 mm/dekad, r = 0.74; May–Sep RMSE 15.9.

# Quality control

- Data audits (6 and 20 Oct, data audit/); the 20 Oct one covered 49 files, 53.7 GB, 34 NetCDF, read lazily: no file is physically corrupt. Defects found: a 370 km² zero-elevation hole in the DEM, a provider-side repeat of three years in eddi2y, an undeclared −4.6×1012 value in the OCO files, unfinished exports (OCO-2, Landsat, ECOSTRESS, two gridMET copies, the last two no longer in the folder) and derived 4 km grids not aligned with gridMET.

- Fix round (20 Oct): SMAP, GFSAD, irrigation, HSG and MOD13A3 notebooks and the USGS 4 km scripts rebuilt →

…_reprocessed_20261020 files with date and version metadata; found on the way that the shared export step skipped 22 % of columns (fixed) and that Earth Engine's MODIS grid is a sphere, not WGS 84. MCD12Q1, HRES-WTD, SSEBop and GRACE share the grid flaw and were not touched.

# Analysis, documents and plots

- TBI_DOCUMENTATION.md (62 kB): VVIP-style vulnerability index and its time-varying extension, with v1–v3 reruns on the USGS data (its XGBoost/LSTM results are superseded). temporal buffering/, XGB model/ and gw_buffering_xgb/ hold the earlier scripts and outputs.

- Groundwater_Buffering_Research_Plan_v2_Ogallala.pdf (20 Oct, 25 pages): three papers; dynamic, vegetation- and ET-driven irrigation masks; a rain-only reference model; 480 m blocks (16×16 Landsat pixels). 26 reference files in gwb papers/.

- plots/ (34 files: shallow/deep groundwater clusters, ECOSTRESS tiles, GDE, MODIS/SSEBop, USGS products) and 7 plotting codes.

# Open items

- Full runs of the 30 m Landsat, OpenET, CDL, LANID and GEDI notebooks, are being completed (a 0.6 GB LANID file is already in merged_datasets).

- Residual-ET full training, then the buffering statistics; unfinished exports and misaligned 4 km grids from the audit.